Alpha 374 BANDPASS ADJUSTMENT PROCEDURE

- Cooling of the 374 - combined with replacement tube prices - is such that we do not suggest exceeding about 400w maximum average total plate dissipation for more than 2-3 seconds. That corresponds roughly to 1 kW average (or 2+ kW PEP) "DC" plate input, or about 1200 W peak output. That was the FCC limit and what they were designed for around 1974.

- Equipment required: electronic cw keyer, peak-reading wattmeter (P-3000 or equivalent), HF transceiver, 240vac Variac.

- Use 240vac from Variac to power amplifier.  All realignment MUST be done with the amplifier in low plate voltage mode (CW), and with the top cover in place to allow airflow over the tubes.  Attempting to adjust the loading capacitors in the bandpass circuit with the amplifier top cover removed will destroy the tubes.  The safe way to do it is to slide back the top cover just far enough to be able to remove the brushed aluminum top cover bar.  This will allow access to the bandpass capacitor shafts.  Then slide the top cover so that it is flush with the top of the front subpanel.  Use masking tape to cover the small slit opening between the cover and the subpanel.  The amplifier primary interlock must be depressed and then the 374 can be turned on.

- Adjustment of LOAD capacitors should be done in the middle of the portion of each band where operation is desired.  Do all loading adjustments into a 50 ohm dummy load ONLY.  Exciter drive should be reduced to zero before keying the amplifier.  Increase drive as required, the capacitor for each band is adjusted to maximum RF output of 750-900w (Ig up to 75-80 ma OK).

- Output of 1.2kw PEP or keyed CW (fast 'dits') should be possible in SSB(HV).  Read power on P-3000 wattmeter.  DO NOT hold the key down for a carrier with amplifier in the SSB(HV) setting with BANDPASS selected!!!!!!!!!

NOTE : For all 76/76A/374A/374/78 amps (untuned inputs) use short coax between transceiver and amplifier, 4 ft or less.

NOTE : Viewed from the front of the amplifier, the five LOAD capacitors for tuning the Bandpass sections are:

10m                                      80m

switch

shaft

15m      20m                         40m

NOTE: To remove front panel for repairs, remove top cross bar at front.  Take out 2 screws on each side of  the rear of chassis that secure side panels.  Entire assembly of side panels and bottom cross bar slides forward and off the amplifier, lifting front panel out of grooves as the sides are moved forward.