Installation of replacement blower motor in Alpha 78/76A/374A/86

The motor bracket is glued to the chassis, and the black fan housing is held in place by the tube deck, a screw, and double-faced tape.  Remove the tubes, tube deck screws and blower housing screw and lift up the tube deck.  Pry the motor bracket off the chassis and lift out the motor and housing together.  Remove the squirrelcage blower from motor shaft to separate the motor and housing.

Scrape off the old foam, glue, and tape from the chassis. Scrape off old tape from the black fan housing.  Put new double-faced tape on bottom and back of housing where the original tape was located.

Mount the new squirrelcage blower on the new motor after putting the motor shaft through the black blower housing.  Glue the motor bracket (with the new foam on the bottom) to the chassis as you set the assembly into the amp.  Use 3m Scotch Adhesive #4475.  Be sure the back of the motor is not touching the chassis.  Do NOT remove the backing of the double-faced tape at this time!!  Put the blower housing screw in place to check the alignment of the housing and motor bracket before the glue sets up.

Allow the glue to dry for 24 hours.  Remove the blower housing screw and gently rotate the housing clockwise to gain access to the backing of the double-faced tape under the left end of the housing.  Remove the plastic backing and carefully lower the housing to the chassis, aligning the hole in the chassis and the fan housing while setting the housing on the chassis.   The tape will secure the two together.

Tip the housing up and counter-clockwise and remove the backing from the tape on the right side of the housing.  Set the housing in place and tighten the screw, lockwasher and nut holding it to the chassis.

Set tube deck in place and replace all tube deck screws.  Install tubes, skirts and chimneys.