91B BIAS LEVEL ADJUSTMENTS

Equipment required:    Digital Multimeter, 14 pin IC clip, clip-lead to short IC clip pins.

Remove amplifier cover and defeat both safety interlocks (microswitch at rear and HV crowbar at center of amp).    Connect the transceiver T/R relay control to the 91B RELAY jack.    Make no connection to the 91B RF input at this time (connect radio RF output to dummy-load or antenna, but NOT to amplifier input).

The following steps will allow adjustment of the tube operating bias switching circuitry:

- Remove knobs and front panel screws to provide access to Control Board adjustments.    Insure that cables between front panel and Control Board are properly seated.

- Set DVM to measure DC voltage.    Connect DVM positive lead to chassis.    Connect negative lead to B- (anode of D16 or D17 on HV Diode Board).

- Connect the 14 pin IC clip to U4.

- Turn on amplifier.

- Place 91B in OPERATE. Short IC clip pins 4 and 5 (this step actually keys the amp simulating RF applied).    Verify 3.8 vdc on DMM, adjust R62 BIAS pot as necessary (this gives 380 ma plate current, the fully turned-on tube operating bias).    Remove short on IC pins.

- Key the transceiver and adjust RF output for zero watts, verify 0.5-0.7 vdc on DMM (50-70 ma plate current); adjust R61 EBS ADJ pot as necessary. Unkey transceiver.

- Check the adjustments in steps 5 and 6 several times, there is some interaction between them.

- With amplifier unkeyed, verify that voltage on U4 pin 6 is 0.50 vdc; adjust R41 EBS LVL pot as necessary.    Remove IC clip.

- When reassembling the amplifier, set both TUNE and LOAD capacitors to have fully meshed plates and install knobs so that TUNE = 100, LOAD = 0.