Here are the initial control settings for tune-up at 18.1 MHz:

Model		           Bandswitch           Tune      Load

PA-76 (ALL)	  		21 	             25		75

PA-374A	 		21 MANUAL           25		75

PA-78	       			21 MANUAL           15		75

PA-77DX		  	       12-18                195		55

The 10 MHz band has a 200 watt power limitation. Accordingly we

recommend against using your ALPHA on this band.

The ALPHA PA-77, 77Dx/Sx, 78, 76, 76A, 374 and 374A should

not be used on 12m.

The plate choke will overheat and burn on that band.