OMNI-VII

Model 588

One-Plug

High Level Notes

Introduction	3

OMNI-VII One-Plug Tasks	3

Timers	3

TimerEvent150ms	3

MaintenanceTimer	4

Winsock Controls	4

Base Command Winsock (CMDWsock)	4

Receive audio over IP Winsock (RIPWsock)	4

TRansmit over IP Winsock (TRIPWsock)	5

DirectX Events	5

Keyboard / Mouse Events	5

One-Plug Forms / Classes / Basic Modules	5

MainControl (Main.frm)	5

frmMore	6

CINI	6

CWType	6

frmControlIF	6

frmLogging	6

frmMemories	7

frmNetConfig	7

frmPresetMemory	7

frmSerialPassthru	7

frmUSB	8

hidDeclarations	8

ini.bas	8

MMbas.bas	8

ResetForm	8

Serial.bas	8

Volume.bas	8

USB and the 610	8

Conditional Compilation Flags	9

ENABLELOGGING	9

TRACEMICIN	9

TRACEMICOUT	9

TRACETRIPOUT	9

ENABLETRIPCOUNTER	9

TRACERIPIN	10

TRACESPEAKERBUFFER	10

ENABLEAUDIOITEMCOUNTTRACKING	10

ENABLEUPDATEDISPRIPAUDIOITEMS	10

SOUNDWRITEPLAYFAILLOGGING	10

ALLOWRIPWRAPADJUSTMENT	10

# Introduction

This file serves as a collection of high level notes for the OMNI-VII One-Plug GUI. It is intended to help someone new to the One-Plug code understand the major concepts in the design of One-Plug.

Written in Microsoft Visual Basic 6.0.

Version Number is defined at “Project” “OMNIVII Properties” “Make tab”.

Several Conditional Compilation Arguments reside on that same location that can assist in debugging different audio buffering paths. Several of them are set to 1 for the current beta test version. This version could be released as 1.0.8 as is. It would give the customer ways to adjust his sound card frequency of the PC and how to handle when the RIP buffer wraps around. These items have actually given a few beta testers a better transmit experience.

Whereas the code was written for Visual Basic, it won’t compile well for RealBasic or other basic programs mainly because it wasn't written to be ANSI Basic compatible. RealBasic would give the resultant exe the ability to be run on different platforms.

This code has been found to work on various Windows OSs, 98SE, ME, 2000, XP Home, XP Pro, VISTA, with various SoundBlaster compatible sound cards, and various Ethernet controllers. The underlying command/query protocol started from the protocol used in serial for Jupiter (538).

See http://www.rfsquared.com/588 for the programmers reference guide for the details of the protocol.

# OMNI-VII One-Plug Tasks

The software is written for a method/event/timer driven Windows operating system. Meaning that there is no infinite loop, just a set of events that are invoked based on Timers, Winsock Network Events, DirectX events, and Keyboard / Mouse Events.

## Timers

Care was taken in defining the rates for the timers. The reason for this is because Visual Basic in run-time doesn’t do a good job of handling multiple interrupts that go off at the same time. It is likely that when a timer interrupt and a DirectX Event trigger simultaneously, sometimes one of them will not go into a pending state and will be lost. Therefore in the case of timers, the rates are not exactly as specified. E.g. the 150ms timer is not 150ms, but as of the writing of this document, it is 148ms. So you will see in the code that when a timer value is reset, it is not an even round number, but a non-multiple of other events/timers rates.

The two timers used in One-Plug are found in the MainControl Main.frm and are defined as follows:

### TimerEvent150ms

This timer is set for a fixed interval of approximately 150ms. See the form properties for the exact interval. As of the writing of this document it was set to 148ms.

TimerEvent150ms has four purposes:

- Handle customer press/hold of the Main Frequency Up/Down and the Sub Frequency Up/Down events.

- Handle the tasked sending of the entire settings from the PC to the rig.  Helps ensure that the settings get to the radio by not sending the setup in one instance.

- Handle sending buffered CWType text from the PC to the rig. Throttles the activity by looking at the bytes available as reported by the rig.

- Maintains the TRIP command to the rig when TRIP is enabled. Helps turn On and Off and continues the TRIP event more reliable.

### MaintenanceTimer

The rate for this timer varies depending up on the state of the software.

Generically it is a 500ms timer, but when states change in the software it is advantageous to make this timer occur earlier, so you will see instances where it is set to 25ms, and another instance where it is set to 500/2ms.

MaintenanceTimer has the following purposes:

- The main purpose of the MaintenanceTimer is to maintain RIP data coming from the rig by periodically sending out a subsequent RIP command. If the rig does not get a RIP command within xx seconds then it will stop sending the RIP audio. So the MaintenanceTimer is the keep-alive event to keep RIP occurring.

- Maintains the TUNING State when tuning is active. (Tuner on/off/power level etc.)

- If the user changed power level, then this timer requests the various power levels.

- Periodically handles the queries for the Monitor S-Unit and Power level feature.

- Periodically handles the queries for the Monitor Rig Settings feature.

- Updates rig statistics so that real-time execution requirements are minimized on the PC for updating text boxes in order to free up PC resources for DirectX events and UDP events that are time critical.

- Handles the maintenance of the state of One-Plug to One-Plug connections.

## Winsock Controls

Winsock is used as the interface to the Ethernet. The Ethernet protocol is based on UDP. UDP packets are received and handled by One-Plug through xx _DataArrival events. One-Plug utilizes “.SendData” methods to send commands over the Ethernet.

Three different UDP events/methods are used based on using three different UDP ports for different purposes. These could be consolidated, but it is easier to have them as separate events taking advantage of the UDP packet number as a key to what the data packet includes, and easier to handle with different _DataArrival routines.

The UDP packet numbers are numbered as:

- Base Command UDP Port = x, where 49152 <= x <= 65520

- Note: This port must be an even numbered port due to how some routers handle UDP data. Some block odd ports without an even pairing for control.

- RIP Port = x+2

- TRIP Port = x+4

The Winsocks used in One-Plug are found in the MainControl Main.frm and are defined as follows:

### Base Command Winsock (CMDWsock)

This Winsock is used for sending commands / queries and for retrieving answers to queries. This is also referred to as the CMDWsock in the code itself. All commands are sent out through SendEthernetCommand, and all queries are sent out through SendEthernetQuery. This provides for tracking and handling demo mode, etc.

Commands are fire and forget, meaning command is sent and no response occurs back from the rig.

Queries are sent on the Ethernet using the base command UDP port number using the CMDWsock. The radio then replies back using this same UDP port number.

### Receive audio over IP Winsock (RIPWsock)

This Winsock is used for receiving audio from the radio. This audio can be the normal receive audio that the radio would play back on the radio’s speaker when in receive mode, or it could be the transmitted audio as would be heard during transmit with monitor turned on.

RIP On is the only exception to the general concept that all commands are sent on the CMDWsock. For RIPing, the RIPWsock is used to send the RIP On/Off command. The radio then starts streaming RIP audio to the UDP Port it received in the RIP on command, in this case it will be the RIPWsock.

### TRansmit over IP Winsock (TRIPWsock)

This Winsock is used for sending transmitted audio from the PC to the radio for transmit over the air.

TRIP On is actually part of the RIP command. Therefore the TRIP On/Off command is sent in the RIP On/Off command packet, and the radio knows then that it should be ready to receive transmit audio packets in on the TRIPWsock UDP port, meaning RIPWsock UDP Port +2.

## DirectX Events

DirectX is used as the interface to the computers sound card. DirectX is a standard Windows component that should be loaded and running on all Windows based computers. I have found some laptops that are not “DirectX” compatible. One such customer received an error trying to launch One-Plug and his answer back from Toshiba on the Model A205-S5804 was that "this model doesn't support DirectX".

DirectXEvent8_DXCallback is the call back routine used to capture audio from the sound cards microphone buffer and send it to the radio for transmit when the customer is trying to transmit voice. It is also the routine that is used to take RIP audio data received by the RIPWsock_DataArrival routine and then sends it to the sound card for playback on the speakers.

## Keyboard / Mouse Events

Keyboard/Mouse events are the main method the customer controls One-Plug and subsequently the radio parameters is by using the keyboard and the mouse. This is mostly self-explanatory in that the forms have certain fields that can be changed, turned on/off, or invoked by using the mouse or by typing in data from the keyboard.

Most functions are simple mouse click events that cause parameters to change. Some of the forms items (those with the “white” background) can be increased by clicking on the right mouse button, and decreased by clicking on the left mouse button. Page up/down, cursor up/down/left/right, home, end, all have distinct uses. For more definitions on what the different keyboard keys can do, look at the One-Plug Users manual.

Most forms have a “PARK” button that is used to place the focus once a given button functions has been done. Coupled with the policy that form buttons/textboxes/etc. all have tabbing disabled, should result in the tab key not taking one from a given menu item to another. The reason I did this was because some people wanted it to go from x to j, whereas others wanted it to go from x to y. So I eliminated its use and everyone seemed OK with that.

# One-Plug Forms / Classes / Basic Modules

Visual Basic is form driven, meaning that you define the user interface in a specific form, that turns into a specific display, and the buttons/sliders that are added to that given form give certain functions. One-Plug utilizes several forms to accomplish the task of controlling the radio remotely. In some instances code is written and placed into basic (.bas) modules that are used by multiple forms or for specific purposes.

There is a peculiarity of form entrance and exit that should be resolved some day. At the moment the buttons to go from one form to the other form must be used otherwise form focus variables will not be set correctly and update events could cause the forms to pop up/down erratically. Customers are aware and use the “CWTYPE” button to go to the CWType display, and then select the X to go back to CONSOLE, rather than using the normal Windows transition method of clicking on a given window to jump to it.

The forms/modules used within One-Plug are:

## MainControl (Main.frm)

The main set of code for One-Plug, and also the form definition for the CONSOLE display

The main initialization and set up for the DirectX events, sound card, and initializes the Winsock controls for the UDP packet handling. The call back routines for the sound card handler and the data arrival handlers for the UDP packets. Loads the frmNetConfig form to get the NETCONFIG display so that the customer can choose the IP/UDP/Passcode information, or run the program in Demo Mode only. The Winsock controls and the timers are seen at the bottom of the form in Visual Basic. This forms size gets set at startup of the program, and can be adjusted by the user.

## frmMore

Provides the form and code for items that use to be on the MainControl CONSOLE display but were moved when the number of parameters was more than a Visual Basic form would allow.

Consists of the following:

- Statistics regarding connection.

  - RIP Statistics for packets received, packets dropped, pc dropped packets.

  - Command Statistics showing how many commands have been sent, and how many queries have been sent, and how many responses to queries have been received.

- A Tone Transmit Generator

- A button to Start Continuous Monitoring of a rig. Helpful in showing if your rig is being used or not, or if the s-unit show a signal is seen, etc.

- A button to jump to the Serial Controls form for serial passthru and serial echo features.

- The sound card fs adjustment items and the feedbacks for the values effectiveness

- A menu item to adjust what to do when the RIP receives buffer wraps over the data going to the sound card.

Items 5 and 6 are not in the 1.0.7 official release, but can be left as is in the beta and released officially after internal TenTec review for “do these really help?” I have feedback from a few customers that said the sound card FS did help, but the ripwrap didn’t do anything noticeable. I was able to measure differences that showed ripwrap did something better than not having it on a couple of computers, but I wasn’t able to hear the difference in the received audio.

## CINI

Class module for the low-level INI file handlers.

Uses the Get/SetPrivateProfileString interface of windows.

## CWType

Provides the form and code for the CWType feature. Macros can be defined for CWType using this form, and stored to disk in the One-Plug installation directory. The current beta version contains a continuous loop macro that can be defined that will be continually sent when invoked until terminated by the customer.

If the CWDecode functionality gets built into the OMNI-VII, then this would be a good place to display the characters received on the PC.

A lot of code is duplicated for each macro. Could make it a list of macros and then have an index instead of all of this code.

## frmControlIF

Provides the form and code for setting the function keys usage and also for turning on the USB interface.

The function keys that you can define are F1, F2, F3, and Page Up/Down. In the beta version the space bar can also be programmed to either PTT or TX On/Off.

For debug purposes, there is a USB Debug button that can be enabled that will bring up the USB form for debugging USB interfaces, e.g. the 610 Interface.

## frmLogging

Provides the form and code for a debug interface for logging data and activities. Not enabled for production. Enabled in the conditional compile in the project properties.

Provides MANY data paths that can be analyzed and printed out to verify that each byte makes it from point A to the end point Z in the audio chain both inbound and outbound.

## frmMemories

Provides the form and code for managing saved sets of radio settings to “memories” in files on the PC. These can be called up and sent to the radio. These are stored in user .ini files in the One-Plug installation directory.

## frmNetConfig

Provides the form and code for managing the network connection parameters such as IP address, UDP Port, Passcode, etc.

Netconfig has many overlayed frames. There are frames that are used in One-Plug to OMNI-VII modes, and also in One-Plug to One-Plug modes.

The mode is defined in the 588radioip.ini file.

The mode is selectable if you enable frmOnePlugMode.and then chkCnx2OMNIVII is used to select an OMNI-VII or another One-Plug. When toggled, then may frames in NETCONFIG appear and others disappear.

Parameters are written to 588radioip.ini in the One-Plug installation directory. These parameters are recalled upon starting One-Plug. The Passcode is only stored if you click the “Store Passcode” button.

This form also has several test/debug buttons to the right of the normal window width. They can be used to force the radio into the loader mode, force a master reset, force read version, retreive system info, and force transmit based on manually setting the transmit bit, TRIP bit, and RIP bit.

## frmPresetMemory

Provides the form and code for recalling preset memories. The user defines the naming on the preset buttons in the memory form.

## frmSerialPassthru

Provides the form and code for the serial pass thru and serial echoes functions.

The serial passthru feature can be demonstrated using this form.

- Connect a serial cable to the OMNI-VII and run procomm or another terminal program on another PC.

- On the Serial Passthru display of One-Plug, type into the text boxes any character string in the Serial Passthru frame.

- Click on Serial Send.

- Watch the text come out on the terminal program on the other computer (or the same computer, doesn’t matter)

Customers have asked that we build a way for the Serial Passthru data could be redirected to come from a virtual COM port on this computer. Whereby a separate logging or control program that runs serial only could be used to control rotors amps and other such devices without having to port them into an Ethernet protocol.

Customers have asked that we build into this display a way to record macros, and call them back later, and a way to enter binary/hexadecimal data. E.g. placing a <<>> around 01, and then the value 0x01 would be sent out on the serial port.

The serial echo feature can be demonstrated by using this form.

- Connect a serial cable to the OMNI-VII and run procomm or another terminal program on another PC.

- Click on “Serial Echo Is Off” in the Serial2Ethernet frame of the Serial Passthru form.

- Then start typing text into the terminal program.

- Watch the text come out in the textbox in the Serial2Ethernet frame.

Customers have asked that we build a way for this data to be made available to a virtual COM port so that a logging program running on the same computer as One-Plug could log the activities.

## frmUSB

Provides the form and code for managing the network connection parameters such as IP address, UDP Port, Passcode, etc. It also will double as the entire connection display when running in One-Plug to One-Plug mode.

## hidDeclarations

Declarations used in USB HID interfacing

## ini.bas

The low-level routines to read/write value to/from the INI files.

## MMbas.bas

The public declaration of the profile string accesses routines for the INI function/files.

## ResetForm

Provides an intermediate dialog box for the user so that they can choose to do a Master Reset or not.  Basically an intermediate “Are You Sure” interface before resetting the rig.

## Serial.bas

Provides the send serial routines for serial pass through and serial echo modes

## Volume.bas

Provides the routines to interface with the mixer.

DESIGNERS NOTE

There are more functions that this program can provide than can be seen with the forms as delivered for production. There are a lot of debug conditional compile options that can be turned on that will bring up more diagnostics and tools. Most forms also have more fields than are shown in the standard form size. The MainControl form is the only form that sets its own size in the code. The rest set their sizes in the properties box, not in code. So if you drag the forms sizes bigger in Visual Basic, most will reveal more features that you can use for testing and tracing issues.

e.g. frmLogging, and frmUSB.

frmLogging can be shown by making the cmdShowLog command button on the MainControl CONSOLE visible. It can be used to log what communications are taking place, and for seeing RIP data. Be careful, enabling this log does reduce performance of the RIPping/TRIPping. But it is a good way to test out functions to see if they occur, the values, when, etc.

frmUSB can be shown by making the cmdUSB command button on the frmControlIF form visible.

It can be used to debug USB events with the TenTec USB Keyer

# USB and the 610

The 610 is set up as a USB HID Keyboard. Which means we don’t have to write a special driver for the PC side. When you “CW Key” in a pattern, the 610 will convert it to a keyboard scan code and send that to the computer just like a standard keyboard would.

When you have ENABLE610 in One-Plug set to TRUE, you will be able to control the Sidetone Frequency, Keyer Speed and Weighting in the 610 Keyer when you adjust these parameters in the One-Plug GUI. With ENABLE610 set to TRUE, you will see a new “USB Keyer Control” frame in the CONTROLS form. The Enable box instructs One-Plug to look for the 610 Keyer PID/VID. The state of the 610 connection is reported in that same frame. Then, when you change the Keyer items mentioned then they will be sent to the 610 device via feature reports.  That is the only reason to have the USB code in One-Plug. If you don’t want to control those features then it doesn’t matter, just disable Enable610.

A good way to test the 610 interface is to simply open up a standard text file in notepad or any text editor and then start keying into the 610. You will see the letter for the keys that you key into the 610 appear in the text file display.

# Conditional Compilation Flags

There are several conditional compilation flags available for use in the “Project” “OMNIVII Properties” “Make Tab” “Conditional Compilation Arguments” text field. The only reason this is mentioned is because there are extensive debug utilities built into One-Plug that take up a lot of coding but provide excellent analysis tools for data paths within the program itself.

Many of the debugging items will require ENABLELOGGING set to 1 in order to make the given debug item viewable in the logging form.

Note: They also enable buttons in the logging form. There are so many buttons for each one that it is possible to enable two of the report buttons that will invoke overlapping buttons in the logging form. So until you get use to which ones do what, enable one at a time and see what/where it goes. It was attempted to be able to use the buttons that represent a given signal path at the same time, so they are positioned on the display such. But not all paths are valid at the same time.

## ENABLELOGGING

Default = 0.

Set to 1 to enable the logging form and all that implies. Lots of logging is possible. Be sure to set back to zero before you release for production.

## TRACEMICIN

Default = 0.

Set to 1 to trace the Microphone incoming data from the microphone buffer.

## TRACEMICOUT

Default = 0.

Set to 1 to trace the Microphone output through the buffers internally.

## TRACETRIPOUT

Default = 0.

Set to 1 to trace the data that is sent to the radio through the TRIP buffer.

So, with TRACEMICIN=1, TRACEMICOUT=1, and TRACETRIPOUT=1, you can trace each data byte from start to finish as it relates to internal to One-Plug as it relates to transmit.

## ENABLETRIPCOUNTER

Default = 0.

Set to 1 so that a menu item will pop up so that you can replace the MIC-in data with a counter. It permits you to trace a specific value and train of packets through One-Plug and then verification of the received data within the OMNI-VII.

## TRACERIPIN

Default = 0.

Set to 1 to capture the RIP data that is received by the RIP Winsock routine. Use to verify that given data sent by the rig makes it to One-Plug. Set up a counter in the OMNI-VII instead of sending audio and this can then be looked at easily to see if One-Plug is dropping packets or if it is skipping a byte, or etc.

## TRACESPEAKERBUFFER

Default = 0.

Set to 1 to capture the RIP data that is going to go to the speaker buffer as placed by the DirectXEvent8_DXCallback event. Combined with TRACERIPIN it can show validity of signal path.

## ENABLEAUDIOITEMCOUNTTRACKING

Default = 1.

Allows you to keep track of audio packets for statistical purposes.

## ENABLEUPDATEDISPRIPAUDIOITEMS

Default = 1.

Enables the display for RIP statistics.

## SOUNDWRITEPLAYFAILLOGGING

Default = 0.

Logs to the logging form when the write and play pointers overlap in the DirectX sound card hard internal buffer.

## ALLOWRIPWRAPADJUSTMENT

Default = 1

Enables the RIP Wrap adjustment menu item in the MORE form.

Set to 0 if this feature is not desired.